# EEG Beta Power Analysis Pipeline

A lightweight EEG analysis pipeline for extracting beta-band power from trial-level data and linking spectral features to behavioral outcomes. Built for the CruX SSVEP BCI project using OpenBCI hardware.

## Overview

This pipeline takes aligned EEG + behavioral data and produces publication-ready figures for conference posters. It covers the full analysis flow from raw epoching through statistical comparison and visualization.

## Pipeline Steps

1. Epoch — Chops continuous EEG into trial windows time-locked to stimulus onset (−200 ms to +500 ms). Each epoch carries a label: stimulus shown, key pressed, and reaction time.

2. Artifact Rejection — Peak-to-peak amplitude thresholding removes epochs contaminated by eye blinks or muscle noise. Threshold is configurable (default 100 µV).

3. FFT / Spectral Analysis — Welch periodogram extracts band power (theta, alpha, beta, gamma) per epoch. Beta power (13–30 Hz) is the primary feature of interest.

4. Behavioral Linking — Three analyses run automatically:

- Pearson correlation between beta power and reaction time

- Independent samples t-test comparing correct vs missed trial beta power

- Block-by-block beta trend to track fatigue across the session

5. Group Comparison — Placeholder comparison between neurofeedback participants and the control baseline (brian1control). Runs a t-test once additional participant data is collected.

6. Visualization — Saves a 6-panel figure including beta power distribution, beta–RT scatter with regression, correct vs missed violin plot, fatigue trend, PSD with band shading, and ERP waveforms.

## Output

eeg_pipeline_results.png   # 6-panel analysis figure

Console prints correlation coefficients, t-test results, and per-block beta means on every run.

## Usage

pip install numpy pandas scipy matplotlib

python3 eeg_pipeline.py

Place eeg_trial_data.csv in the same directory. To use real OpenBCI data, replace the simulated raw array in __main__ with your MNE Raw object and event onsets.

## Data Format

The pipeline expects a CSV with one row per trial:

| Column | Description |
| --- | --- |
| trial_id | Trial index |
| block | Block number (fatigue grouping) |
| stimulus_letter | Letter shown on screen |
| key_pressed | Participant's response |
| correct | 1 = hit, 0 = miss |
| reaction_time_ms | RT in milliseconds (blank if missed) |
| beta_power_db | Beta band power in dB |
| alpha_power_db | Alpha band power in dB |
| theta_power_db | Theta band power in dB |
| trial_onset_s | Stimulus onset time in seconds |
| participant | Participant ID |

## Baseline Results (brian1control, n=80 trials)

- Beta–RT correlation: r = −0.304, p = 0.011

- Correct trial beta: 12.96 dB vs missed: 11.79 dB

- Accuracy: 87.5%

- Beta trend: mild increase across blocks (12.27 → 14.01 dB), consistent with fatigue

What the visualization shows (using simulated brian1control data, 80 trials):

The six panels map directly to your research questions:

- Beta Power Distribution — histogram centered around ~12.8 dB with a dashed mean line. This becomes your baseline for comparing against neurofeedback participants.

- Beta Power vs RT — scatter with color-coded blocks showing the negative correlation (r = −0.304, p = 0.011). Higher beta power predicts faster reaction times, which is the core result you're hunting for.

- Beta: Correct vs Missed — violin plot. Correct trials show higher beta (μ = 12.96 dB) vs missed (μ = 11.79 dB), meaning the spectral fingerprint of successful detection is already visible.

- Beta Over Blocks — error-bar trend showing mild beta increase across 4 blocks, which is your fatigue marker over the session.

- Power Spectral Density — Welch periodogram with band shading (theta/alpha/beta/gamma). Shows the alpha peak at ~10 Hz and the beta bump. This is a poster-ready panel.

- ERP: Correct vs Missed — time-locked to stimulus onset at 0 ms. The correct trials show a bigger negative deflection around 280 ms (like an N2/P3 complex), which is the neural signature of successful target detection.

The pipeline script (eeg_pipeline.py) is structured as 6 clearly labeled functions that you can slot your real MNE Raw + Events data into — just replace the simulated array generation with raw.get_data() and your actual event onsets. The CSV schema matches what the alignment step should produce once that gate is cleared.